Ephys Inc. Master Analysis Protocol

Last updated: 2025_12_22 by Christian Althaus and Rachel Callaghan

****Things to add

-Work flow protocol with abf saving and storage

-roughly which abf files correspond to which traces and measures

-

-Metadata workflow and input

-Tips and tricks

-“updated” for each section

Resting Membrane Potential (RMP)

- Drag and drop file into clampfit

- Set Cursor 1 and 2 at 0.0s and .200s, respectively

- Hit Statistics

- Select Baseline as “Fixed at: 0”

- Select search region to 1

- Range: Cursor 1…2

- “From” and “To” should reflect 0.0 and 0.200s

- Measurements Select mean (mV)

- Destination option: append results to sheet

- Hit ok

- Mean values will be displayed in the Results page under R1S1 mean (mV)

- Repeat for all RMP files across groups

- TIP: if you drag and drop the next file onto the one you just analyzed, it will place the cursors at 0.0 to .200s instead of their default starting points

- Copy Results table into excel or analysis software to compile and average

Input Resistance and Sag Ratio

- Drag and drop abf file onto clampfit

- Cursor Positions

  - Double click on analysis window and under “cursor options” add another pair of cursors for a total of 3 sets

  - double click on cursor 1 and type in Time the seconds below – can scroll to next one by hitting “next”

- 0.0 s

- 0.2 s

- 0.3 s

- 0.5 s

- 1.0 s

- 1.2 s

- Note*** if one of the traces has an action potential in the baseline region (cursors 1 and 2) or the steady state region (cursors 5 and 6), remove that trace prior to running the statistics below.

- Click the “Statistics” symbol at top of window

- Setup Measurements (see figures below):

  - Under “Trace Selection”

    - Select “All visible signals” and “All visible traces”

  - Under “Peak Polarity”

    - Check “Apply to all Signals”

    - Select “Positive-going”

  - Under “Baseline region”

    - Select “fixed at:” and enter 0mV

  - Search region 1

    - Range: Cursors 1 and 2

    - Location should self populate

    - Measurements: Check “MEAN”

  - Search region 2

    - Range: Cursors 3 and 4

    - Measurements: Check “Antipeak amplitude” AND “Time of Antipeak”

  - Search region 3

    - Cursors 5 and 6

    - Measurement: Check “MEAN”

  - Under “Destination Option” Choose to append or replace results in sheet (can keep track of previous recording’s measurements or just replace the old ones – your decision)

  - Column order: Measurement, Region, Signal

  - See figures below

Search Region 1:

Search region 2: 				Search region 3:

- Click “OK”

- Measurements will appear in the Results window under the Statistics tab

- Copy and paste this table into excel to analyze

- Columns of interest are (see figure below):

  - Note: Region refers to area between cursors.  Signal refers to mV vs pA (stimulus)

  - R2S1 (Region2, Signal 1) Antipeak Amp (mV)

    - This is your peak negative mV for each trace between cursors 3 and 4

    - Subtract this value from R1S1 Mean (baseline) to get the change, or delta, in mV

  - You can largely ignore the next 3 columns, but time of anti peak could be interesting to see at what point in time the peak Sag is reached

  - R1S1 Mean (mV)

    - This is your baseline mV between cursors 1 and 2

    - Subtract this value FROM your Antipeak and R3S1 Mean (steady state) to get the delta mV

  - R1S2 Mean (pA)

    - Baseline pA from your stimulus signal

    - Subtract this value FROM your R3S2 Mean to get the delta pA, or in other words, how much current was injected for that stimulus

  - File path

    - Can ignore or save.  Up to you

Calculate Input Resistance

- Subtract the Baseline mV and pA values (Region 1) from the corresponding Steady State values (Region 3)

- Graph the resulting columns on an I/V scatter plot

  - pA on the x-axis

  - mV on the y-axis

- Add a best fit line to the graph

  - Only graph sweeps between -50 and +50 pA

- The slope of the line will be your Input Resistance in Ohms!

- Multiply by 1000 to get the units to MOhms

- (note: graph below is different example from clampfit values above)

Copied over from Clampfit into Input Resistance Analysis Template (excel)

Calculate the Sag Ratio

- Subtract the Baseline mV value (R1S1 Mean) from the Antipeak Amp (R2S1) to get the delta peak mV (Sag or hump Amplitude)

- This Sag or hump amplitude could also be worth measuring and graphing

- Sag Ratio = (Steady State delta mV) / (Antipeak delta mV)

- Divide the mV column calculated from the input resistance by the antipeak column just calculated to get the Sag Ratio

- We’re only really interested in the two most negative current injections (-300 pA and -200 pA) – which are each performed twice (traces 1, 2, 4, 5)

- Average the two for each injection and that is your Sag ratio for that cell (see example below)

- Note: the smaller the Sag Ratio, the larger the Sag.  The more negative current injections will likely result in a smaller sag ratio (larger Sag) due to more Ih channels being activated

5 Spike AHP

Spike AHP

- Drag and drop .abf file into Clampfit

- Adjust Cursor Positions

  - In analysis window, right click and select “properties”

  - Under “Cursor Options” increase the number of cursor pairs to 4

  - Double click cursor 1 and change “Time (s)” to the value listed below – move through all cursors adjusting their value by selecting “next”

Cursors (newly updated 2025_03_17 for fewer decimal places)

Baseline

- 0.0

- 0.2

Medium (peak)

- 0.38

- 0.58

200ms AHP

- 0.57

- 0.59

slowAHP (sAHP)

- 1.37

- 1.39

- Click the “Statistics” symbol at the top of the window

- Set up measurements

  - Under “Trace Selection”

    - Select “Cmd0” and “All visible traces”

  - Under “Peak Polarity”

    - Uncheck “Apply to all signals” and just select “Cmd0”

    - Select “Positive-going”

  - Under “Baseline region”

    - Select “fixed at:” and enter 0mV

  - Search region 1

    - Range: Cursors 1 and 2

    - Location should self populate

    - Measurements: Check “MEAN”

  - Search region 2

    - Range: Cursors 3 and 4

    - Measurements: Check “Antipeak amplitude”

  - Search region 3

    - Cursors 5 and 6

    - Measurement: Check “MEAN”

  - Search region 4

    - Cursors 7 and 8

    - Measurement: Check “MEAN”

  - Under “Destination Option” Select either append or replace results in sheet, matter of personal preference

  - Under “Column Order” Select “Measurement, Signal, Region)

- Click “Ok”

- Measurements will be printed in the “Results” window

- Copy and paste data into an excel spreadsheet for further analysis

- An Explanation of each column of the produced results

  - Column 1: The file name, this can be especially useful if you are compiling the results of multiple recordings

  - Column 2: The Trace/Sweep that the results were produced from

  - Column 3: The time at which the Trace/Sweep began (ms)

  - Column 4: S1R2 Antipeak Amp, the magnitude of the signal’s antipeak (negative peak)

  - Column 5: S1R1 Mean, is the average of the cell’s BASELINE, which is used to normalize the other readings for AHP calculation

  - Column 6: S1R3 Mean, is the average unnormalized value of the mAHP

  - Column 7: S1R4 Mean, is the averaged unnormalized value of the sAHP

  - Column 8: File Path, is the file path used to find the origin of the .abf file used to produce these results

Calculate AHP

- The purpose of the data calculations is to normalize the values present in column 4, 6, and 7 as well as averaging the values of the two traces

  - Subtract the S1R1 mean from S1R2 the Antipeak value to calculate peakAHP

  - Subtract the S1R1 mean from the S1R3 mean to calculate the mAHP

  - Subtract the S1R1 mean from S1R4 mean to calculate the sAHP

  - After these calculations are done for both traces, calculate the average of the baseline, peakAHP, mAHP, and sAHP for each recording from the two traces

25 Spike AHP

| 25 spike peak/m/sAHP cursor placement |  |  |  |
| --- | --- | --- | --- |
|  |  | seconds | rounded update |
| baseline | 1 | 0.00000 | 0 |
|  | 2 | 0.20000 | 0.2 |
| Medium (peak) | 3 | 0.66394 | 0.66 |
|  | 4 | 0.86394 | 0.86 |
| 200ms (old medium) | 5 | 0.85394 | 0.85 |
|  | 6 | 0.87394 | 0.87 |
| slow | 7 | 1.65394 | 1.65 |
|  | 8 | 1.67394 | 1.67 |

50 Spike AHP

Cursors (DMG_1/2/26):

Baseline

- 0.0

- 0.2

Medium (peak)

- 1.41

- 1.61

200ms AHP

- 1.60

- 1.62

slowAHP (sAHP)

- 2.40

- 2.42

1s AP Firing

Cursors

- 0s and 0.2s

- 0.290s and1.5s

OBSOLETE ANALYSIS PROTOCOL IMAGES