Rheobase Protocol

- Drag and drop abf file into clampfit

- Set cursor 1 to 0s

- Manually place cursor 2 where the FIRST action potential threshold takes place

- Click statistics

- Select search to region 1

- Range: Cursor 1…2

- From (ms) should always reflect 0.

- To(ms) will always be different depending on where Cursor 2 was manually set

- Destination Option

  - Append results to sheet

- Column Order: Measurement, Region, Signal

- Measurements:

  - Check Peak Amplitude and Antipeak amplitude

- Results will be added to Results Sheet

- Continue to do this for all Rheobase Files

- When all files are complete, proceed to next step

- Copy and paste results from clampfit to Rheobase Template excel under the “Clampfit Export” section within the excel template

  - It will automatically do the calculations

Calculations defined:

- R1S1 Peak: The Action Potential Threshold mV

- R1S2 peak- R1S2 antipeak: Δ pA